Hochschule Ravensburg-Weingarten

University of Applied Sciences

Specification of

RISC-V 64-Bit Single-Instruction Microcontroller

Author: Yash Balu Sangale

Matriculation No.: 29346383

Study Program: M.Eng. Electrical Engineering and Embedded Systems

Faculty of Electrical Engineering and Computer Science

Supervisor: Prof. Dr.-Ing. A. Siggelkow

Revision: 1.1

Date: January 22, 2026

# Contents

Contents	1

Chapter 1 — Introduction	1

1.1 Purpose of Document	1

1.2 Scope	1

1.3 Reference Documents	1

Chapter 2 — Product Overview	1

2.1 Top Level Description	1

2.1.1 Introduction	1

2.1.2 Key Features	1

2.2 Architecture Concepts: Overview	1

2.2.1 Functional Block Diagram	1

2.2.2 Package	1

2.3 System View	1

2.3.1 Application System Description and Use Cases	1

2.3.2 System Description	1

2.3.3 System Integration and Application Circuit	1

2.3.4 Compliances	1

2.4 Architecture Concepts Overview	1

2.4.1 Technology	1

2.4.2 System Memory Concept	1

2.4.3 Software Architecture	1

2.4.4 Interprocessor Communication Concept	1

2.5 Functional Block Overview	1

2.6 Pin List	1

Chapter 3 — Architecture Concepts	1

3.0.1 Technology	1

3.0.2 CPU Concept (Updated for FPU)	1

3.0.3–3.0.11	1

Chapter 4 — Description of the Design Elements	1

4.1 CPU Core	1

Chapter 9 — Floating-Point Unit (FPU) Specification	1

9.1 Introduction	1

9.2 Compliance and Standards	1

9.3 Floating-Point Data Formats	1

9.3.1 Double-Precision Format (IEEE 754-2008 binary64)	1

9.3.2 Single-Precision Format (IEEE 754-2008 binary32)	1

9.3.3 Special Values	1

9.4 Floating-Point Register File	1

9.4.1 Register Organization	1

9.4.2 Floating-Point Control and Status Register (fcsr)	1

9.5 Rounding Modes	1

9.6 Exception Handling	1

9.6.1 IEEE 754-2008 Exception Flags	1

9.6.2 Default Results for Exceptional Conditions	1

9.7 Supported Instructions	1

9.7.1 Floating-Point Arithmetic Instructions (RV64D)	1

9.7.2 Floating-Point Load and Store Instructions	1

9.7.3 Floating-Point Comparison Instructions	1

9.7.4 Floating-Point Conversion Instructions	1

9.7.5 Floating-Point Move and Sign-Injection Instructions	1

9.7.6 Floating-Point Classify Instruction	1

9.8 FPU Microarchitecture	1

9.8.1 Pipeline Integration	1

9.8.2 Execution Latencies	1

9.8.3 FP Register File	1

9.8.4 Operand Forwarding	1

9.8.5 NaN Handling	1

9.8.6 Subnormal Number Handling	1

9.9 Hardware Interfaces	1

9.9.1 Interfaces to CPU Core	1

9.9.2 Interfaces to FP Register File	1

9.9.3 Clock and Reset	1

9.9.4 Voltage Domain	1

9.10 Floating-Point CSR Registers	1

9.11 Address Map and Integration	1

9.12 Power Considerations	1

9.13 Verification and Test Strategy	1

9.13.1 Unit-Level Simulation	1

9.13.2 IEEE 754-2008 Compliance Testing	1

9.13.3 Integration-Level Testing	1

9.13.4 Regression Testing	1

9.14 Future Extensions	1

Chapter 5 — Toolchain	1

5.1 Introduction of the Toolchain	1

5.2 Technologies Used	1

5.2.1 Build and Automation Technologies	1

5.2.2 Simulation and Verification Tools	1

5.2.3 Development and Integration Tools	1

5.3 Usage of the Toolchain	1

5.3.1 Terminal-Based Usage	1

5.3.2 VS Code Extension Usage	1

5.4 Further Development of the Toolchain	1

Chapter 6 — Test And Debug	1

Chapter 7 — Validation and Test	1

Chapter 8 — Electrical Characteristics	1

8.1 Operating Conditions	1

# Chapter 1 — Introduction

Table 1.1: History / Revision – Chapter 1

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | January 22, 2026 | Y. Sangale | Initial template draft |

## 1.1 Purpose of Document

This document defines the first specification of the Reduced Instruction Set Computer - Five (RISC-V) single-instruction-bit microcontroller, covering architectural, electrical, and functional aspects. Revision 1.1 adds the Floating-Point Unit (FPU) specification as an additional design element.

## 1.2 Scope

The specification outlines the design decisions, Instruction Set Architecture (ISA) subset, interfaces, and architectural details of the RISC-V System-on-Chip. This revision extends the scope to include the IEEE 754-2008 compliant double-precision FPU.

## 1.3 Reference Documents

- RISC-V Unprivileged ISA Specification, Volume I

- RISC-V Privileged Architecture Specification

- IEEE 754-2008: Standard for Floating-Point Arithmetic

- RWU "Specification Document Preparation Guidelines"

# Chapter 2 — Product Overview

Table 2.1: Revision History – Chapter 2

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | January 22, 2026 | Y. Sangale | Initial Draft |
| 1.1 | January 22, 2026 | Y. Sangale | Added FPU to Key Features |

## 2.1 Top Level Description

### 2.1.1 Introduction

The designed microcontroller is a 64-bit RISC-V (RV64I) based single-core processor aimed at basic embedded applications. It operates at an internal clock frequency of 80 MHz and provides key interfaces such as General-Purpose Input/Output (GPIO), Universal Asynchronous Receiver-Transmitter (UART), and Quad Serial Peripheral Interface (QSPI) for communication and JTAG for uploading programs on the controller. The architecture follows the Harvard Architecture model with separate instruction and data memories. A Wishbone-based interconnect provides standardized communication between the core and peripherals.

The peripheral subsystem includes a configurable GPIO controller with interrupt capability, a UART for serial communication, and a QSPI interface for high-speed memory or peripheral access. The system integrates a clock generation block capable of supplying phase-shifted or divided clocks to peripherals. Programming and debugging are supported through a JTAG interface. The interrupt controller supports up to eight sources with programmable trigger modes (rising, falling, both, pulse). The design now incorporates an IEEE 754-2008 compliant double-precision Floating-Point Unit (FPU) supporting the RV64D extension.

### 2.1.2 Key Features

The RISC-V System-on-Chip microcontroller includes the following key features:

- RISC-V RV64I + RV64D ISA (with FPU)

- Single-core, single-pipeline processor (scalar, in-order)

- Harvard Architecture with separate I-MEM and D-MEM (64 KB each)

- 80 MHz internal main clock with peripheral clock dividers

- Wishbone bus architecture for peripheral connectivity

- 32 × 64-bit general-purpose integer registers (x0–x31) and 64-bit program counter

- 32 × 64-bit floating-point registers (f0–f31)

- IEEE 754-2008 compliant double-precision FPU (RV64D extension)

- 8 configurable interrupt lines (IRQ7:0) with vector-based ISR dispatch

- Memory-mapped peripherals for modular extensibility

- UART, GPIO, QSPI interfaces

- JTAG for programming and on-chip debug access

## 2.2 Architecture Concepts: Overview

### 2.2.1 Functional Block Diagram

Functional blocks diagram will be included in the final version of this document.

The architecture consists of a processing subsystem (CPU + FPU), peripheral subsystem, power management unit, and external interface controllers. Each block communicates via a shared system bus.

### 2.2.2 Package

Package type and pin configuration details will be provided in the final version of this document.

## 2.3 System View

### 2.3.1 Application System Description and Use Cases

The system is intended for use in low-power embedded applications such as IoT sensors, wearables, and smart home devices. The addition of the FPU extends the range of applicable use cases to include:

- Educational demonstrations of IEEE 754 floating-point arithmetic, RISC-V F/D extension execution, and FPU pipeline behavior.

- FPGA-based prototyping of embedded systems requiring floating-point computation (e.g., sensor fusion, signal processing).

- Control-oriented applications requiring trigonometric, exponential, or high-precision arithmetic.

- Research or coursework involving FPU microarchitecture, rounding modes, and exception handling.

### 2.3.2 System Description

The system is organized as a Harvard Architecture microcontroller with separate instruction and data memories, unified through a Wishbone interconnect for all data accesses. The CPU implements the RV64I ISA and features a straightforward single-pipeline structure. With the addition of the FPU (RV64D), the CPU pipeline is extended with a dedicated floating-point execution stage that shares the register file interface but operates on the separate 64-bit floating-point register bank (f0–f31).

### 2.3.3 System Integration and Application Circuit

For FPGA deployments, the system integrates cleanly with minimal external circuitry. The FPU is tightly integrated into the CPU pipeline and requires no additional external components. Key considerations remain unchanged: stable 80 MHz clock, proper I/O pin constraints, and optional QSPI flash for program storage.

### 2.3.4 Compliances

The FPU is designed for full compliance with IEEE 754-2008 for double-precision operations. No formal certification is mandated for FPGA-based prototype use. For commercial deployment, compliance with IEEE 754-2008 must be verified through formal simulation and exhaustive corner-case testing.

## 2.4 Architecture Concepts Overview

### 2.4.1 Technology

The microcontroller is intended for implementation in a modern 90 nm CMOS technology node. The FPU design uses fully synchronous combinational/sequential logic compatible with automated place-and-route flows. The FPU area overhead is estimated at approximately 15–20% of the total core area, consistent with typical double-precision FPU implementations at this technology node.

### 2.4.2 System Memory Concept

The memory concept is unchanged from Revision 1.0. Floating-point load/store instructions (FLD, FSD) use the same D-MEM data path as integer load/store operations. The floating-point register file (f0–f31) is internal to the CPU/FPU and is not memory-mapped.

### 2.4.3 Software Architecture

The software architecture extends the bare-metal development flow to include floating-point support. The RISC-V GCC toolchain is configured with -march=rv64imd to enable double-precision FPU code generation. Floating-point programs follow the same compilation and JTAG-based loading flow as integer programs.

### 2.4.4 Interprocessor Communication Concept

Unchanged. The design is single-core with no interprocessor communication.

## 2.5 Functional Block Overview

The microcontroller now includes the following updated functional blocks:

- CPU Core (RV64IM): Single-issue scalar pipeline (integer).

- FPU (RV64D): IEEE 754-2008 double-precision floating-point unit, tightly coupled to the CPU pipeline. Implements all required RV64D instructions including arithmetic, compare, convert, and load/store.

- Instruction Memory (I-MEM): 64 KB program memory.

- Data Memory (D-MEM): 64 KB data RAM, shared by integer and floating-point load/store.

- Wishbone Interconnect: Single-master peripheral bus.

- GPIO Controller, UART Controller, QSPI Controller, Interrupt Controller, Peripheral Clock Generator, JTAG Interface — unchanged from Revision 1.0.

## 2.6 Pin List

The FPU is fully internal and does not add external pins. The pin list is unchanged from Revision 1.0.

# Chapter 3 — Architecture Concepts

Table 3.1: Revision History – Chapter 3

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | January 22, 2026 | Y. Sangale | Initial Draft |
| 1.1 | January 22, 2026 | Y. Sangale | Added FPU architecture concept |

This section defines the fundamental architectural principles of the 64-bit RISC-V single-core microcontroller. Revision 1.1 extends the architecture to include the double-precision Floating-Point Unit (FPU).

### 3.0.1 Technology

Unchanged. 90 nm CMOS target. The FPU adds approximately 15–20% area overhead and has no impact on the core clock frequency target of 80 MHz due to pipelined execution.

### 3.0.2 CPU Concept (Updated for FPU)

The CPU core is extended with a tightly coupled FPU implementing the RISC-V D-extension (double-precision floating-point). The combined CPU+FPU supports:

- Pipeline: Fetch – Decode – Execute – (FP Execute) – Writeback

- ISA Support: RV64I + M-extension + D-extension (double-precision FP)

- Integer Datapath: 64-bit ALU, 32 × 64-bit integer registers (x0–x31)

- Floating-Point Datapath: 64-bit FPU, 32 × 64-bit FP registers (f0–f31)

- FP Register File: Separate from integer register file; not memory-mapped

- Floating-Point Status/Control Register (fcsr): Contains rounding mode (frm) and accrued exception flags (fflags)

- FP Memory Access: FLD/FSD instructions share D-MEM data path with integer load/store

### 3.0.3–3.0.11

Bus, Memory, Interrupt, Clock, System Control, Register, Power, Security, and Debug concepts are unchanged from Revision 1.0. The FPU does not introduce new bus masters, memory regions, or interrupt lines.

# Chapter 4 — Description of the Design Elements

Table 4.1: Revision History – Chapter 4

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | January 22, 2026 | Y. Sangale | Initial Draft |
| 1.1 | January 22, 2026 | Y. Sangale | Added FPU design element (Section 4.2) |

## 4.1 CPU Core

Unchanged from Revision 1.0. See previous sections for CPU Core hardware interfaces, bus interface, registers, functional overview, structural overview, interrupts, clock/reset, and voltage class.

# Chapter 9 — Floating-Point Unit (FPU) Specification

Table 9.1: Revision History – Chapter 9

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | January 22, 2026 | Y. Sangale | Initial FPU Specification Draft |

## 9.1 Introduction

This chapter specifies the Floating-Point Unit (FPU) added to the RV64I_RWU microcontroller as an integral extension. The FPU implements the RISC-V D-extension (RV64D) and is fully compliant with the IEEE 754-2008 Standard for Floating-Point Arithmetic in double-precision (64-bit) format. The FPU is tightly coupled to the CPU pipeline, sharing the instruction fetch and decode stages, and operates on a dedicated 64-bit floating-point register file (f0–f31) separate from the integer register file.

The FPU is designed for deterministic, in-order execution consistent with the single-pipeline philosophy of the microcontroller. All floating-point operations are performed in hardware with no software emulation fallback. The FPU supports all mandatory IEEE 754-2008 operations, all five rounding modes, and all five floating-point exception flags.

## 9.2 Compliance and Standards

The FPU is designed to comply with the following standards and specifications:

| Standard / Specification | Version / Reference | Compliance Level |
| --- | --- | --- |
| IEEE 754-2008 | Standard for Floating-Point Arithmetic | Full (double-precision) |
| RISC-V Unprivileged ISA | Volume I, D-Extension (RV64D) | Full |
| RISC-V Unprivileged ISA | Volume I, F-Extension (RV64F) | Full (subset of D) |
| RISC-V Privileged ISA | fcsr register (frm + fflags) | Full |

## 9.3 Floating-Point Data Formats

### 9.3.1 Double-Precision Format (IEEE 754-2008 binary64)

The primary data format supported by the FPU is the 64-bit double-precision binary floating-point format as defined in IEEE 754-2008:

| Field | Bits | Description |
| --- | --- | --- |
| Sign (S) | 1 | 0 = positive, 1 = negative |
| Exponent (E) | 11 | Biased exponent, bias = 1023 (0x3FF) |
| Mantissa / Fraction (M) | 52 | Fractional part of significand (implicit leading 1 for normal numbers) |
| Total | 64 | 64-bit double-precision floating-point word |

### 9.3.2 Single-Precision Format (IEEE 754-2008 binary32)

The FPU also supports single-precision (32-bit) operations as required by the RISC-V F-extension (RV64F), which is a prerequisite of RV64D. Single-precision values are stored NaN-boxed in the upper 32 bits of the 64-bit floating-point registers when held in the FP register file.

| Field | Bits | Description |
| --- | --- | --- |
| Sign (S) | 1 | 0 = positive, 1 = negative |
| Exponent (E) | 8 | Biased exponent, bias = 127 (0x7F) |
| Mantissa / Fraction (M) | 23 | Fractional part of significand |
| Total | 32 | 32-bit single-precision; NaN-boxed to 64-bit in FP registers |

### 9.3.3 Special Values

| Value | Exponent (biased) | Mantissa | Description |
| --- | --- | --- | --- |
| +/- Zero | 0x000 | 0 | Positive and negative zero; equal in comparison |
| Denormal / Subnormal | 0x000 | Non-zero | Gradual underflow; no implicit leading 1 |
| +/- Infinity | 0x7FF | 0 | Result of overflow or divide-by-zero (unless trapped) |
| Quiet NaN (qNaN) | 0x7FF | Bit[51]=1 | Propagates through arithmetic without signalling |
| Signalling NaN (sNaN) | 0x7FF | Bit[51]=0, non-zero | Triggers Invalid Operation exception on use |
| Canonical NaN | 0x7FF | 0x8000000000000 | RV64D canonical NaN value (0x7FF8000000000000) |

## 9.4 Floating-Point Register File

### 9.4.1 Register Organization

The FPU provides 32 dedicated 64-bit floating-point registers, named f0 through f31. These registers are distinct from the 32 integer registers (x0–x31) and are not accessible via integer load/store instructions. Each register holds one double-precision (64-bit) or one NaN-boxed single-precision (32-bit) value.

| Register | ABI Name | Description / Convention |
| --- | --- | --- |
| f0–f7 | ft0–ft7 | Floating-point temporaries (caller-saved) |
| f8–f9 | fs0–fs1 | Floating-point saved registers (callee-saved) |
| f10–f11 | fa0–fa1 | Floating-point function arguments / return values |
| f12–f17 | fa2–fa7 | Floating-point function arguments |
| f18–f27 | fs2–fs11 | Floating-point saved registers (callee-saved) |
| f28–f31 | ft8–ft11 | Floating-point temporaries (caller-saved) |

### 9.4.2 Floating-Point Control and Status Register (fcsr)

The fcsr is a 32-bit RISC-V CSR (address 0x003) that contains two sub-fields accessible individually via fflags (CSR 0x001) and frm (CSR 0x002):

| Field | Bits in fcsr | CSR Address | Description |
| --- | --- | --- | --- |
| fflags – NX (Inexact) | Bit 0 | 0x001 [4:0] | Set when result is inexact |
| fflags – UF (Underflow) | Bit 1 | 0x001 [4:0] | Set when result underflows |
| fflags – OF (Overflow) | Bit 2 | 0x001 [4:0] | Set when result overflows |
| fflags – DZ (Divide by Zero) | Bit 3 | 0x001 [4:0] | Set on division by zero |
| fflags – NV (Invalid Operation) | Bit 4 | 0x001 [4:0] | Set on invalid operation (e.g., NaN input) |
| frm – Rounding Mode | Bits [7:5] | 0x002 [2:0] | Active rounding mode (see Section 9.5) |

## 9.5 Rounding Modes

The FPU implements all five rounding modes defined by IEEE 754-2008, as required by the RISC-V D-extension. The active rounding mode is read from the frm field of fcsr, unless overridden by a per-instruction dynamic rounding mode field (rm field in the instruction encoding).

| Encoding (frm) | RISC-V Mnemonic | IEEE 754-2008 Mode | Description |
| --- | --- | --- | --- |
| 3'b000 | RNE | roundTiesToEven | Round to nearest, ties to even (default) |
| 3'b001 | RTZ | roundTowardZero | Round toward zero (truncate) |
| 3'b010 | RDN | roundTowardNegative | Round toward negative infinity (floor) |
| 3'b011 | RUP | roundTowardPositive | Round toward positive infinity (ceiling) |
| 3'b100 | RMM | roundTiesToAway | Round to nearest, ties away from zero |
| 3'b111 | DYN | — | Use dynamic rounding mode from fcsr.frm |

## 9.6 Exception Handling

### 9.6.1 IEEE 754-2008 Exception Flags

The FPU supports all five IEEE 754-2008 exception types. Exceptions are sticky (write-1-to-clear) and are recorded in the fflags field of the fcsr register. No hardware trap is generated by default; all exceptions are handled via software inspection of fflags after the operation. This is consistent with the RISC-V model which does not mandate hardware trapping of FP exceptions.

| Exception | Flag Bit | Trigger Condition | IEEE 754-2008 Reference |
| --- | --- | --- | --- |
| Invalid Operation (NV) | fflags[4] | Operation on NaN, 0/0, Inf-Inf, sqrt of negative | Section 7.2 |
| Divide by Zero (DZ) | fflags[3] | Non-zero dividend divided by zero | Section 7.3 |
| Overflow (OF) | fflags[2] | Result magnitude exceeds representable range | Section 7.4 |
| Underflow (UF) | fflags[1] | Result is subnormal and inexact | Section 7.5 |
| Inexact (NX) | fflags[0] | Result is rounded; exact result not representable | Section 7.6 |

### 9.6.2 Default Results for Exceptional Conditions

When an exception occurs and is not trapped, the FPU returns the IEEE 754-2008 defined default result:

- Invalid Operation: Canonical quiet NaN (0x7FF8000000000000 for double)

- Divide by Zero: Signed infinity (+/-Inf)

- Overflow: Signed infinity or largest representable value, depending on rounding mode

- Underflow: Subnormal result (gradual underflow); zero if below minimum subnormal

- Inexact: Correctly rounded result

## 9.7 Supported Instructions

### 9.7.1 Floating-Point Arithmetic Instructions (RV64D)

| Instruction | Mnemonic | Description |
| --- | --- | --- |
| Add (double) | FADD.D | fd = fs1 + fs2 (double-precision) |
| Subtract (double) | FSUB.D | fd = fs1 - fs2 (double-precision) |
| Multiply (double) | FMUL.D | fd = fs1 * fs2 (double-precision) |
| Divide (double) | FDIV.D | fd = fs1 / fs2 (double-precision) |
| Square Root (double) | FSQRT.D | fd = sqrt(fs1) (double-precision) |
| Fused Multiply-Add | FMADD.D | fd = (fs1 * fs2) + fs3 |
| Fused Multiply-Sub | FMSUB.D | fd = (fs1 * fs2) - fs3 |
| Fused Neg. Multiply-Add | FNMADD.D | fd = -(fs1 * fs2) - fs3 |
| Fused Neg. Multiply-Sub | FNMSUB.D | fd = -(fs1 * fs2) + fs3 |
| Min (double) | FMIN.D | fd = min(fs1, fs2) per IEEE 754-2008 minNum |
| Max (double) | FMAX.D | fd = max(fs1, fs2) per IEEE 754-2008 maxNum |
| Add (single) | FADD.S | fd = fs1 + fs2 (single-precision) |
| Subtract (single) | FSUB.S | fd = fs1 - fs2 (single-precision) |
| Multiply (single) | FMUL.S | fd = fs1 * fs2 (single-precision) |
| Divide (single) | FDIV.S | fd = fs1 / fs2 (single-precision) |
| Square Root (single) | FSQRT.S | fd = sqrt(fs1) (single-precision) |
| Fused Multiply-Add (S) | FMADD.S | fd = (fs1 * fs2) + fs3 (single) |

### 9.7.2 Floating-Point Load and Store Instructions

| Instruction | Mnemonic | Description |
| --- | --- | --- |
| Load Double | FLD | fd = Memory[rs1 + imm12] (64-bit load to FP reg) |
| Store Double | FSD | Memory[rs1 + imm12] = fs2 (64-bit store from FP reg) |
| Load Word (float) | FLW | fd = Memory[rs1 + imm12] (32-bit load, NaN-boxed) |
| Store Word (float) | FSW | Memory[rs1 + imm12] = lower 32 bits of fs2 |

### 9.7.3 Floating-Point Comparison Instructions

| Instruction | Mnemonic | Description |
| --- | --- | --- |
| Compare Equal (D) | FEQ.D | rd = (fs1 == fs2) ? 1 : 0; sNaN raises NV |
| Compare Less-Than (D) | FLT.D | rd = (fs1 < fs2) ? 1 : 0; NaN raises NV |
| Compare Less-Equal (D) | FLE.D | rd = (fs1 <= fs2) ? 1 : 0; NaN raises NV |
| Compare Equal (S) | FEQ.S | rd = (fs1 == fs2) ? 1 : 0 (single-precision) |
| Compare Less-Than (S) | FLT.S | rd = (fs1 < fs2) ? 1 : 0 (single-precision) |
| Compare Less-Equal (S) | FLE.S | rd = (fs1 <= fs2) ? 1 : 0 (single-precision) |

### 9.7.4 Floating-Point Conversion Instructions

| Instruction | Mnemonic | Description |
| --- | --- | --- |
| Convert W to D | FCVT.D.W | fd = (double) rs1 (32-bit signed integer to double) |
| Convert WU to D | FCVT.D.WU | fd = (double) rs1 (32-bit unsigned integer to double) |
| Convert L to D | FCVT.D.L | fd = (double) rs1 (64-bit signed integer to double) |
| Convert LU to D | FCVT.D.LU | fd = (double) rs1 (64-bit unsigned integer to double) |
| Convert D to W | FCVT.W.D | rd = (int32) fs1 (double to 32-bit signed integer) |
| Convert D to WU | FCVT.WU.D | rd = (uint32) fs1 (double to 32-bit unsigned integer) |
| Convert D to L | FCVT.L.D | rd = (int64) fs1 (double to 64-bit signed integer) |
| Convert D to LU | FCVT.LU.D | rd = (uint64) fs1 (double to 64-bit unsigned integer) |
| Convert S to D | FCVT.D.S | fd = (double) fs1 (single to double) |
| Convert D to S | FCVT.S.D | fd = (float) fs1 (double to single, NaN-boxed) |

### 9.7.5 Floating-Point Move and Sign-Injection Instructions

| Instruction | Mnemonic | Description |
| --- | --- | --- |
| Move D to Integer | FMV.X.D | rd = bit-cast of fd to integer (no conversion) |
| Move Integer to D | FMV.D.X | fd = bit-cast of rs1 to double (no conversion) |
| Move W to FP | FMV.W.X | fd = NaN-box(rs1[31:0]) (single-precision bit move) |
| Move FP to W | FMV.X.W | rd = sign-extend(fd[31:0]) |
| Sign Inject (D) | FSGNJ.D | fd = ¦fs1¦ with sign of fs2 |
| Neg. Sign Inject (D) | FSGNJN.D | fd = ¦fs1¦ with negated sign of fs2 |
| XOR Sign Inject (D) | FSGNJX.D | fd = ¦fs1¦ with sign = sign(fs1) XOR sign(fs2) |

### 9.7.6 Floating-Point Classify Instruction

| Instruction | Mnemonic | Description |
| --- | --- | --- |
| Classify (D) | FCLASS.D | rd = 10-bit mask classifying fs1 (see Table 9.8) |
| Classify (S) | FCLASS.S | rd = 10-bit mask classifying fs1 (single-precision) |

Table 9.8 — FCLASS bit encoding:

| Bit | Meaning |
| --- | --- |
| 0 | rs1 is negative infinity |
| 1 | rs1 is a negative normal number |
| 2 | rs1 is a negative subnormal number |
| 3 | rs1 is negative zero |
| 4 | rs1 is positive zero |
| 5 | rs1 is a positive subnormal number |
| 6 | rs1 is a positive normal number |
| 7 | rs1 is positive infinity |
| 8 | rs1 is a signalling NaN |
| 9 | rs1 is a quiet NaN |

## 9.8 FPU Microarchitecture

### 9.8.1 Pipeline Integration

The FPU is tightly coupled to the CPU pipeline. Floating-point instructions follow the same Fetch–Decode pipeline stages as integer instructions. After decode, FP instructions are dispatched to the FPU execution unit. The pipeline is extended with a dedicated FP Execute stage, which may require multiple clock cycles for latency-sensitive operations (divide, square root). In-order execution is maintained; the pipeline stalls (via hazard detection) while multi-cycle FP operations complete.

| Pipeline Stage | Description |
| --- | --- |
| Fetch (IF) | Instruction fetched from I-MEM, shared with integer pipeline |
| Decode (ID) | Instruction decoded; FP instruction dispatched to FPU; operands read from FP register file |
| FP Execute (FPX) | FPU performs the operation; 1–N cycles depending on operation type |
| Writeback (WB) | Result written back to FP register file or integer register file (for FMV/FCLASS/compare) |

### 9.8.2 Execution Latencies

The following table provides target execution latencies for the initial FPU implementation. Multi-cycle operations cause pipeline stalls.

| Operation | Target Latency (clock cycles) | Notes |
| --- | --- | --- |
| FADD.D, FSUB.D | 3 | Includes alignment and rounding |
| FMUL.D | 4 | Double-precision multiplier |
| FMADD.D / FMSUB.D | 5 | Fused multiply-add (single rounding) |
| FDIV.D | 20–24 | Iterative divider (SRT or Goldschmidt) |
| FSQRT.D | 25–30 | Iterative square root |
| FADD.S, FSUB.S | 2 | Single-precision |
| FMUL.S | 3 | Single-precision |
| FDIV.S | 12–16 | Single-precision division |
| FCVT.* | 2–3 | Conversion latency |
| FLD / FSD | 1 (+ memory latency) | D-MEM single-cycle access |
| FMV.X.D / FMV.D.X | 1 | No arithmetic |
| FEQ, FLT, FLE | 1 | Comparison result to integer register |
| FCLASS | 1 | Classification to integer register |

### 9.8.3 FP Register File

The floating-point register file contains 32 registers, each 64 bits wide. It provides two read ports and one write port, consistent with the single-issue pipeline operation. The register file is synchronous and clocked at 80 MHz.

### 9.8.4 Operand Forwarding

Result forwarding from the FPU writeback stage to the FPU execute stage is supported to reduce stalls when the result of one FP instruction is immediately consumed by the next. Forwarding is implemented for all single-result operations. Multi-cycle operations (FDIV, FSQRT) require the pipeline to stall until the result is available.

### 9.8.5 NaN Handling

All NaN propagation rules follow IEEE 754-2008 Section 6.2. The FPU propagates quiet NaN inputs to the output without raising an exception. Signalling NaN inputs cause the Invalid Operation (NV) exception flag to be set and the result is the canonical quiet NaN. When multiple NaN inputs are present, the canonical NaN is returned.

### 9.8.6 Subnormal Number Handling

The FPU fully supports subnormal (denormal) numbers as required by IEEE 754-2008. Subnormal inputs and outputs are handled in hardware without software emulation. Gradual underflow is implemented; no flush-to-zero mode is used in the default configuration.

## 9.9 Hardware Interfaces

### 9.9.1 Interfaces to CPU Core

| Signal | Direction | Width | Description |
| --- | --- | --- | --- |
| fpu_op[5:0] | CPU → FPU | 6 | FPU operation code from instruction decoder |
| fpu_rm[2:0] | CPU → FPU | 3 | Rounding mode (from instruction or fcsr.frm) |
| fpu_rs1[63:0] | CPU → FPU | 64 | Operand 1 (from FP register file or integer reg) |
| fpu_rs2[63:0] | CPU → FPU | 64 | Operand 2 (from FP register file) |
| fpu_rs3[63:0] | CPU → FPU | 64 | Operand 3 for fused operations (FMADD/FMSUB) |
| fpu_valid | CPU → FPU | 1 | Indicates a valid FP operation is presented |
| fpu_result[63:0] | FPU → CPU | 64 | FP result to be written to FP register file or integer reg |
| fpu_fflags[4:0] | FPU → CPU | 5 | Exception flags generated by this operation |
| fpu_ready | FPU → CPU | 1 | FPU result is ready (used for pipeline stall control) |
| fpu_int_result[63:0] | FPU → CPU | 64 | Integer result for FMV.X.D, FCLASS, compare ops |

### 9.9.2 Interfaces to FP Register File

The FPU reads operands directly from the floating-point register file. The register file interface provides two asynchronous read ports and one synchronous write port, clocked at 80 MHz. Write-enable is asserted by the FPU when a valid result is ready.

### 9.9.3 Clock and Reset

The FPU is clocked by the same 80 MHz CLK_MAIN signal as the CPU core. It is reset synchronously by the system RESET_N signal. On reset, all internal FPU pipeline registers are cleared, and fcsr is reset to 0x00000000 (rounding mode = RNE, all exception flags cleared).

### 9.9.4 Voltage Domain

The FPU operates in the same 3.3 V logic domain as the CPU core.

## 9.10 Floating-Point CSR Registers

| CSR Address | Name | Width | Access | Reset Value | Description |
| --- | --- | --- | --- | --- | --- |
| 0x001 | fflags | 32 | RW | 0x00000000 | Floating-Point Accrued Exceptions (fflags[4:0]) |
| 0x002 | frm | 32 | RW | 0x00000000 | Floating-Point Dynamic Rounding Mode (frm[2:0]) |
| 0x003 | fcsr | 32 | RW | 0x00000000 | Combined FP Control/Status: {frm[2:0], fflags[4:0]} |

Reading fflags or frm individually is equivalent to reading the corresponding bit fields of fcsr. Writing to fflags or frm updates the corresponding bits in fcsr atomically.

## 9.11 Address Map and Integration

The FPU is an internal CPU component and does not occupy any memory-mapped address range. It is not visible on the Wishbone peripheral bus. All interaction with the FPU occurs through RISC-V ISA instructions and the fcsr/fflags/frm CSRs. There are no additional external pins introduced by the FPU.

## 9.12 Power Considerations

The FPU is always powered in this initial implementation. Clock gating is not implemented in Revision 1.0 of the FPU. Power optimization (clock gating when no FP instruction is in flight) is planned for a future revision. The estimated additional dynamic power dissipation of the FPU at 80 MHz is approximately 5–15% above the base CPU core, depending on the floating-point instruction mix.

## 9.13 Verification and Test Strategy

### 9.13.1 Unit-Level Simulation

Each FPU sub-unit (adder, multiplier, divider, sqrt, converter) is to be verified independently with a dedicated SystemVerilog testbench. Test vectors include normal, subnormal, infinity, and NaN inputs across all rounding modes. All five exception conditions are verified to be correctly detected and reported.

### 9.13.2 IEEE 754-2008 Compliance Testing

Compliance is verified using the Berkeley TestFloat suite (or an equivalent RISC-V compliant FP test suite), which provides exhaustive test vectors for IEEE 754-2008 double-precision operations. The test suite covers:

- All rounding modes for each arithmetic operation

- Boundary cases: overflow, underflow, exact zero, maximum and minimum representable values

- All NaN types (qNaN, sNaN) and NaN propagation

- Subnormal operands and subnormal results

- Correctly rounded square root and division

### 9.13.3 Integration-Level Testing

Full instruction-level tests are executed using the RV64I toolchain. Firmware test programs compiled with -march=rv64imd validate each RV64D instruction mnemonic, result correctness, and fcsr flag behavior within the actual microcontroller simulation environment (Vivado XSIM).

### 9.13.4 Regression Testing

All FPU test cases are integrated into the CTest regression framework. Tests are categorized as:

- fpu_unit_tests: Individual operation correctness per data format

- fpu_ieee_compliance: TestFloat-derived compliance vectors

- fpu_integration: Full RV64D program execution tests

- fpu_corner_cases: NaN, Inf, subnormal, and boundary value tests

## 9.14 Future Extensions

The following extensions are planned for future revisions of the FPU:

- Flush-to-Zero (FTZ) mode: Optional mode to replace subnormal results with zero for performance in applications where subnormal handling is not critical.

- RISC-V Q-extension: Quadruple-precision (128-bit) floating-point support as a future optional extension.

- FPU Clock Gating: Power optimization by gating the FPU clock when no floating-point instruction is in the pipeline.

- Hardware exception trapping: Optional hardware trap on FP exception, triggered via a CSR configuration register.

- Transcendental functions: Hardware-accelerated sin, cos, exp, and log as custom extensions (non-standard).

# Chapter 5 — Toolchain

Table 5.1: Revision History – Chapter 5

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | January 22, 2026 | Y. Sangale | Initial template draft |

The toolchain is centered around a modular CMake-based build system and is complemented by a custom-developed Visual Studio Code (VS Code) extension. With the addition of the FPU, the RISC-V GCC toolchain must be invoked with -march=rv64imd and -mabi=lp64d to enable double-precision floating-point code generation.

## 5.1 Introduction of the Toolchain

For the RV64I_RWU project, a comprehensive toolchain was designed and extended to support the full development lifecycle of a 64-bit RISC-V processor core including the FPU extension. The primary goals are reproducible and cross-platform builds, automated firmware compilation and hardware simulation, scalable regression testing using CTest, reduced onboarding effort for new developers, and tight integration between software, hardware, and verification flows.

## 5.2 Technologies Used

### 5.2.1 Build and Automation Technologies

- CMake (>= 3.20): Primary build system generator with modular architecture for auto-discovery of RTL, firmware, and test files.

- CTest: Automated unit, integration, firmware, and IP-level regression testing with parallel execution.

- RISC-V GNU Toolchain: riscv64-unknown-elf-gcc with -march=rv64imd for integer + double-precision FP firmware compilation.

### 5.2.2 Simulation and Verification Tools

- Xilinx Vivado XSIM: RTL simulation for both console-based automation and GUI waveform visualization.

- TCL Scripts: Standardized waveform setup, simulation control, and batch execution.

### 5.2.3 Development and Integration Tools

- Visual Studio Code: Primary development environment.

- Node.js: Custom VS Code extension development.

- Git: Version control.

- Sphinx: HTML and PDF documentation generation.

## 5.3 Usage of the Toolchain

### 5.3.1 Terminal-Based Usage

Configuration and Build:

cmake -S . -B build -G "MinGW Makefiles"

cmake --build build

### 5.3.2 VS Code Extension Usage

The custom rwu-rv64i-toolchain VS Code extension provides build, simulation, and regression testing commands. Target names are automatically inferred from the currently opened file.

## 5.4 Further Development of the Toolchain

- Support for multiple simulators beyond XSIM

- Improved dependency visualization and reporting

- Enhanced cross-compilation support for additional RISC-V variants

- Integration with CI pipelines

- Graphical dashboards for build and test status in VS Code

# Chapter 6 — Test And Debug

Table 6.1: Revision History – Chapter 6

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | January 22, 2026 | Y. Sangale | Initial Draft |

This section will be completed later. It will describe how the SoC (including the FPU) will be tested or simulated for functionality, including details on testbenches, simulation tools, and debugging interfaces. FPU-specific test strategies are outlined in Chapter 9 (Section 9.13).

# Chapter 7 — Validation and Test

Table 7.1: Revision History – Chapter 7

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | January 22, 2026 | Y. Sangale | Initial Draft |

This section will be completed later. It will list the memory map and all registers with their addresses and reset values in tabular format, including all peripheral registers, CPU control and status registers (CSRs), and floating-point CSRs (fflags, frm, fcsr at addresses 0x001, 0x002, 0x003).

# Chapter 8 — Electrical Characteristics

Table 8.1: Revision History – Chapter 8

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | January 22, 2026 | Y. Sangale | Initial Draft |

This section will be completed later.

## 8.1 Operating Conditions

| Parameter | Typical | Unit |
| --- | --- | --- |
| Supply Voltage | 3.3 | V |
| Clock Frequency | 80 | MHz |
| Operating Temp Range | -40 to +85 | °C |